- Download Install

- Computer IE Web Browser

1、Dome camera connect with the local area network(LAN) or connect with computer directly, make sure the network segment of computer IP is “1”, open the computer IE browser, input default IP: 192.168.1.10, as below picture:

Pic.1

- Refresh the log in interface then user can enter into the log interface, as below picture:

- Open the mobile phone APP, add the camera, click the LAN search device ID; Enter the camera password empty, as shown in the figure;

- Pic.2

- 3. NVR Shortcut Command List

| Function | Mode Of Operation |
| --- | --- |
| The first cruise line（1-16 presets） | call71 |
| The second cruise line（17-32 presets） | call72 |
| The third cruise line（33-48 presets） | call73 |
| The fourth cruise line（240-255 presets） | call74 |
| A cruise preset for 4 seconds | call75 |
| A cruise preset for 8 seconds | call76 |
| A cruise preset for 10 seconds | call77 |
| A cruise preset for 15 seconds | call78 |
| A cruise preset for 20 seconds | call79 |
| To record the first path | call81 |
| To record the second path | call82 |
| To record the third path | call83 |
| To record the fourth path | call84 |
| end of the recording track | call85 |
| Start the first track | call86 |
| Start the second track | call87 |
| Start the third track | call88 |
| Start the fourth track | call89 |
| Set the left limit point A | call91 |
| Set the right limit point B | call92 |
| To enable A, B two points scans | call93 |
| To clear A, B two points scans | call94 |
| Enable the level 360 - degree continuous scanning | call99 |
| A 360 - degree scan, and A, B two scanning speed | Call 65, 66, 67, 68, 69 preset position, the corresponding rate is 3, 6 degrees per second, 9, 15 degrees and 40 degrees |
| Idle task setting | Call 61:Preset level 1 |
|  | Call 62:trajectory 1 |
|  | Call 63:A/B scans |
|  | Call 64:360 degree scans |
| clear all of the presets | call60 |
| Open the camera menu | Call70 |
| Open the dome menu | call95 |
| Restore the factory | call96 |

NOTE: If the speed dome camera without OSD menu, you must NOT to call 95 preset, it will lead to the camera out of control; If do this by mistake, you need power off to restart, or you can no need do any operate until 1 minute,the camera can be return to normal and can cont

Tip:This machine uses 12V; Product configuration power supply For more detailed instructions, please use the CD to read information tips, thank you for your cooperation.