IP Camera CMS User manual (Android Version)

1

IP Camera CMS User Manual (Android Version)

V1.3

IP Camera CMS User manual (Android Version)

2

Content

1. Software description .................................................................................................................3

2. One key to configure WIFI .......................................................................................................3

3. Add Camera ..............................................................................................................................4

3.1.

3.2.

LAN Search...............................................................................................................4

Scan barcode .............................................................................................................4

4. Delete Camera...........................................................................................................................4

5. View the video ..........................................................................................................................5

6. Video Interface illustration........................................................................................................5

7. Camera parameters....................................................................................................................6

7.1.

7.2.

7.3.

7.4.

7.5.

7.6.

7.7.

7.8.

7.9.

7.10.

7.11.

Change password ......................................................................................................6

Motion Detection Alarm setting................................................................................6

Alarm Linkage ..........................................................................................................7

Timing record............................................................................................................7

Audio Setting ............................................................................................................8

Video settings............................................................................................................8

SD Card setting .........................................................................................................9

Device time setting....................................................................................................9

Email Setting.............................................................................................................9

FTP Setting..............................................................................................................10

Device information..................................................................................................10

8. Other settings ..........................................................................................................................11

9. Local pictures..........................................................................................................................11

10.

Video...............................................................................................................................12

Local Video .............................................................................................................12

Video playback........................................................................................................12

10.2.1. Local Video playback......................................................................................12

10.2.2. Online video playback.....................................................................................12

10.1.

10.2.

IP Camera CMS User manual (Android Version)

3

1. Software description

HICHIP IP Camera CMS (Android Version) is a kind of tool to manage network

cameras, can be realized to monitor network camera images and other functions

in LAN and Internet on Android system. It supports search to add ID and manually

enter camera IP in LAN; support one key to configure WIFI, add device, delete and

edit function; support audio monitoring and intercom; support snap photos to

mobile phone; support image quality level Settings, and other functions.

First connect the camera power supply, after about 40s, will hear a prompt

sound, if not heard, please press the RESET button in the equipment base for 10s.

Note: 1. Please use the camera self-contained power adapter, so as to

avoid damage the device

2. Please do not insert net cable before power on;(If done, one key to

configure WIFI will not work)

2. One key to configure WIFI

After install mobile app, mobile phone connected to WIFI, click add camera –

one key configure WIFI -- input WIFI password -- popup weather hear the

camera hint sound, choice yes, system will configure WIFI himself; when WIFI

configure successfully, it will have hint sound.

IP Camera CMS User manual (Android Version)

4

3. Add Camera

3.1. LAN Search

Click

to add camera, enter into add camera interface, type in camera

UID (figure 4, figure 5) :

1. Search: click LAN Search button, search the network camera in the mobile

phone wireless network; Select the UID

2. Enter the camera password(default password is admin).

3. Click Finish on the upper right corner.

3.2. Scan barcode

Through QR code generation tool to generate QR code using this UID, and then

through mobile phone APP to scan the bar code to add

4. Delete Camera

Click Edit, click

，will prompt whether delete camera, click YES

IP Camera CMS User manual (Android Version)

5

5. View the video

After add the camera for a few seconds , waiting the camera connected

automatically, when prompt online instructions, it means can view the camera

video:

Camera status illustration:

Connecting: Now the camera is connecting with network, according to

different network, it costs different time

Online: means the device connected normally, click to view videos

6. Video Interface illustration

：Monitoring off;

：Monitoring on;

：when monitoring icon turns to ，press to realize intercom（You can not

loosen the icon , press it for long time, if loosen, intercom will turn off）

： click this icon, it will snap photos to mobile phone (The photos save to Snapshot file

folder in mobile phone)

：Local Record off

IP Camera CMS User manual (Android Version)

6

：Local Record on

：click this icon, can choose resolution type: HD or smooth

HD: It is connected to first stream

smooth: It is connected to second stream

7. Camera parameters

When camera online, click

, enter into the interface of camera parameter setting

7.1. Change password

can change the access password of camera (only can change administrator password)

7.2. Motion Detection Alarm setting



Open Motion detection: motion detection on or off

IP Camera CMS User manual (Android Version)

7



Motion detection sensitivity:

High, medium and low can be set; High: the sensitivity value is 75; medium

sensitivity value is 50: low sensitivity value is 25

7.3. Alarm Linkage





Alarm Notifications

Click ON,open alarm Notifications;

SD Card record while alarm

Choose on, means the alarm video will save to SD

Card;



Email alarm and send pictures

Choose on, means it will send pictures to specified

mailbox when alarm



save pictures to FTP Server

Choose on, mean it will save alarm pictures to

specified FTP server



save videos to FTP server

Choose on, means alarm videos save to specified

FTP server

7.4. Timing record



Video file length: 15-900seconds can be set,

default is 600seconds



whether video on: on off can be set, default is on

IP Camera CMS User manual (Android Version)

8

7.5. Audio Setting



Input type

Linear input and microphone input can be

set

when audio input type is microphone

input, the input volume and output volume is 80

and 95;

when audio input type is linear input, the

input volume and output volume is 15 and 95;

7.6. Video settings



code rate

The device code rate of first stream and

second stream can be self-define; scope range

32kbps-6144kbps



frame rate

The device frame rate of first stream and second

stream can be self-defined;



Video coding level

1-6 can be set, default is 1; the smaller the value,

the better the image level, the big the stream



Video system

50HZ and 60HZ can be set

IP Camera CMS User manual (Android Version)

9

7.7. SD Card setting

Can obtain the total space of the SD card and the size of the available space, can

format SD card

7.8. Device time setting

1. The device time can synchronization mobile phone time

2. can set device time zone, after setting successfully, the device will restart

7.9. Email Setting

Eg: sending mailbox is QQ mailbox

Smtp server：smtp.qq.com

Server port：465

Secure connection：SSL

user name and password is QQ account and

password

Receive address：the mailbox that receive alarm

pictures

Send address: sender’s mailbox address, if it is

QQ mailbox, need to fill the QQ mailbox address

(the same as user name)

Theme and information can be filled in casually

After completed, click Apply; then click test to

know whether can send alarm email

IP Camera CMS User manual (Android Version)

10

7.10. FTP Setting

FTP server： fill in the FTP address

Server port：fill in FTP port；

fill in the user name and password setted on

FTP server

after completed, click Apply; then click test

to know whether can send pictures to FTP

server

7.11. Device information

can obtain device information: including network status, user connection number, IP

address, Subnet mask, gateway, DNS and so on

IP Camera CMS User manual (Android Version)

11

8. Other settings

 Mirror and Flip

click

click

, can set Mirror and Flip

 Lens zoom

, can set the lens zoom in, zoom out,

focus add, focus reduction and other operations

Note: This function need the lens supporting

zoom

 Preset settings and calls

click

, can set presets and call presets;

Note: This function need the device

supporting PTZ

9. Local pictures

click

, enter into playback interface

1. Local pictures are saved by ID

2. Long press one picture can delete it

IP Camera CMS User manual (Android Version)

12

10. Video

10.1. Local Video

:Click the icon, the icon is red

;In the video icon appears on the right

side,

; Save the path is “VideoRecording” folder of the video;

10.2. Video playback

10.2.1. Local Video playback

Click on the video icon, into the video playback interface, select local, click to

play back the video equipment;Then pops out video list, click on the video playback;If

the phone installed more than one player, please select a player to play.

10.2.2. Online video playback

click

, enter into watching interface

Can playback the videos in SD Card, including

timing video and alarm video